Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#01)

Write Lab Title Here

Student Name:   Abdul Ahad Minhas            Roll No: 24i-6531            Section: CE-A

Date performed: 21 Aug, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

In this task, we applied concepts of all 7 basic gates on Verilog. Here, we initialized three variables and applied test bench and used basic gate function test combinations also get output on simulation of waves.

Code:

//AND GATE module LogicGates(a,b,c); input a; input b; output c; and a1(c,a,b); endmodule module testbench(); reg x; reg y; wire z; LogicGates uut(x,y,z); initial begin x=0; y=0; #50 x=0;y=1; #50 x=1;y=0; #50 x=1;y=1; end endmodule //OR GATE module LogicGates(a,b,c); input a; input b; output c; and a1(c,a,b); endmodule module testbench(); reg x; reg y; wire z; LogicGates uut(x,y,z); initial begin x=0; y=0; #50 x=0;y=1; #50 x=1;y=0; #50 x=1;y=1; end endmodule //NOT GATE module LogicGates(a,b); input a; output b; not a1(b,a); endmodule module testbench(); reg x; wire z; LogicGates uut(x,z); initial begin x=0; #50 x=1; end endmodule //NAND GATE module LogicGates(a,b,c); input a; input b; output c; nand a1(c,a,b); endmodule module testbench(); reg x; reg y; wire z; LogicGates uut(x,y,z); initial begin x=0; y=0; #50 x=0;y=1; #50 x=1;y=0; #50 x=1;y=1; end endmodule //NOR GATE module LogicGates(a,b,c); input a; input b; output c; nor a1(c,a,b); endmodule module testbench(); reg x; reg y; wire z; LogicGates uut(x,y,z); initial begin x=0; y=0; #50 x=0;y=1; #50 x=1;y=0; #50 x=1;y=1; end endmodule //XOR GATE module LogicGates(a,b,c); input a; input b; output c; xor a1(c,a,b); endmodule module testbench(); reg x; reg y; wire z; LogicGates uut(x,y,z); initial begin x=0; y=0; #50 x=0;y=1; #50 x=1;y=0; #50 x=1;y=1; end endmodule //XNOR GATE module LogicGates(a,b,c); input a; input b; output c; xnor a1(c,a,b); endmodule module testbench(); reg x; reg y; wire z; LogicGates uut(x,y,z); initial begin x=0; y=0; #50 x=0;y=1; #50 x=1;y=0; #50 x=1;y=1; end endmodule

Output:

AND GATE OR GATE NOT GATE NAND GATE NOR GATE XOR GATE XNOR GATE

Task 2:

Description:

In this task, we applied concepts of all 7 basic gates on Verilog. Here, we initialized three variables and applied test bench and used basic gate function test combinations also get output on simulation of waves. But, here we used dataflow method in which we assigned variables for each basic gate functions too.

Code:

module LogicGates(a,b,and_out,or_out,not_out,nand_out,nor_out,xor_out,xnor_out); input a; input b; output and_out,or_out,not_out,nand_out,nor_out,xor_out,xnor_out; assign and_out = a&b; assign or_out = a¦b; assign not_out = ~a; assign nand_out = ~(a&b); assign nor_out = ~(a¦b); assign xor_out = a^b; assign xnor_out = ~(a^b); endmodule module testbench(); reg x; reg y; wire and_outt,or_outt,not_outt,nand_outt,nor_outt,xor_outt,xnor_outt; LogicGates uut(x,y,and_outt,or_outt,not_outt,nand_outt,nor_outt,xor_outt,xnor_outt); initial begin x=0; y=0; #50 x=0;y=1; #50 x=1;y=0; #50 x=1;y=1; end endmodule

Output:

Task 3:

Description:

Here, we made all basic logic gate circuits on Logisim. We made circuits of for each gate with input probes and output LED. Further, we tested all four combinations of logic gates for each one.

Code:

Output:

Task 4:

Description:

Here, we applied logic gate concepts on a given circuit which basically giving output of XOR gate. We initialized three variables and then, applied gate functions and created testbench to get desired output.

Code:

module Logic_circuit(a,b,c); input a,b; output c; wire A1; not a1(A1,a); wire B1; not b1(B1,b); wire and1; and c1(and1,A1,b); wire and2; and d1(and2,B1,a); or e1(c,and1,and2); endmodule; module testbench(); reg x; reg y; wire z; Logic_circuit uut(x,y,z); initial begin x=0;y=0; #50 x=0;y=1; #50 x=1;y=0; #50 x=1;y=1; end endmodule;

Output:

Task 5:

Description:

We created circuit of task 4 on Logisim using concepts of logic gates. And tested all 4 combinations of logics and get desired output of XOR gate.

Code:

Output:

Task 6:

Description:

Here, we write code on Verilog for complementing values. We basically initialized 8 variables and 4 are input and other 4 are output variables which actually give us result of complemented variables. Then, made testbench and tested result by simulation.

Code:

module one_complement(a,b,c,d,e,f,g,h); input a,b,c,d; output e,f,g,h; assign e=~a; assign f=~b; assign g=~c; assign h=~d; endmodule module testbench(); reg l,m,n,o; wire p,q,r,s; one_complement uu(l,m,n,o,p,q,r,s); initial begin l=0; m=0; n=0; o=0; #50 l=0; m=0; n=0; o=1; #50 l=0; m=0; n=1; o=0; #50 l=0; m=0; n=1; o=1; #50 l=0; m=1; n=0; o=0; #50 l=0; m=1; n=0; o=1; #50 l=0; m=1; n=1; o=0; #50 l=0; m=1; n=1; o=1; #50 l=1; m=0; n=0; o=0; #50 l=1; m=0; n=0; o=1; #50 l=1; m=0; n=1; o=0; #50 l=1; m=0; n=1; o=1; #50 l=1; m=1; n=0; o=0; #50 l=1; m=1; n=0; o=1; #50 l=1; m=1; n=1; o=0; #50 l=1; m=1; n=1; o=1; #50; end endmodule

Output: