Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#02)

Write Lab Title Here

Student Name: Abdul Ahad Minhas               Roll No: 24i-6531             Section: CE-A

Date performed: 25 Aug, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1(Gate level):

Description:

In this task, we created half and full adder by gate level system. I initialized inputs and outputs and then, made functions for each gate applied to create half adder and and combining 2 half adders getting a full adder.

Code:

module half_adder(a,b,z,s,c,S,c2,C); input a,b,z; output s,c,S,c2,C; xor sum1(s,a,b); and carry1(c,a,b); xor sum2(S,s,z); and carry(c2,s,z); or carry2(C,c,c2); endmodule module testbench(); reg x,y,z; wire A,B,C,D,E; half_adder HA(x,y,z,A,B,C,D,E); initial begin x=0; y=0; z=0; #50 x=0; y=0; z=1; #50 x=0; y=1; z=0; #50 x=0; y=1; z=1; #50 x=1; y=0; z=0; #50 x=1; y=0; z=1; #50 x=1; y=1; z=0; #50 x=1; y=1; z=1; #50; end endmodule

Output:

Task 1(DATA Flow):

Description:

In this task, we created half and full adder by Data Flow system. I initialized inputs and outputs and then, made functions for each gate applied to create half adder and and combining 2 half adders getting a full adder.

Code:

module half_adder(a,b,z,s,c,S,c2,C); input a,b,z; output s,c,S,c2,C; xor sum1(s,a,b); and carry1(c,a,b); xor sum2(S,s,z); and carry(c2,s,z); or carry2(C,c,c2); endmodule module testbench(); reg x,y,z; wire A,B,C,D,E; half_adder HA(x,y,z,A,B,C,D,E); initial begin x=0; y=0; z=0; #50 x=0; y=0; z=1; #50 x=0; y=1; z=0; #50 x=0; y=1; z=1; #50 x=1; y=0; z=0; #50 x=1; y=0; z=1; #50 x=1; y=1; z=0; #50 x=1; y=1; z=1; #50; end endmodule

Output:

LogiSim Output :

Task 2(Data Flow):

Description:

In this task, I used Data Flow system then, created 3X8 decoder. By initializing 3 inputs and 8 outputs for decoders and each specific is getting me true on specific input.

Code:

module decoder(a,b,c,d0,d1,d2,d3,d4,d5,d6,d7); input a,b,c; output d0,d1,d2,d3,d4,d5,d6,d7; assign d0 = ~a&~b&~c; assign d1 = ~a&~b&c; assign d2 = ~a&b&~c; assign d3 = ~a&b&c; assign d4 = a&~b&~c; assign d5 = a&~b&c; assign d6 = a&b&~c; assign d7 = a&b&c; endmodule module testbench_decode(); reg x,y,z; wire a,b,c,d,e,f,g,h; decoder dec(x,y,z,a,b,c,d,e,f,g,h); initial begin x=0; y=0; z=0; #50 x=0; y=0; z=1; #50 x=0; y=1; z=0; #50 x=0; y=1; z=1; #50 x=1; y=0; z=0; #50 x=1; y=0; z=1; #50 x=1; y=1; z=0; #50 x=1; y=1; z=1; #50; end endmodule;

Output:

Task 2(Gate level):

Description:

In this task, I used Gate level system then, created 3X8 decoder. By initializing 3 inputs and 8 outputs for decoders and each specific is getting me true on specific input.

Code:

module decoder(a,b,c,d0,d1,d2,d3,d4,d5,d6,d7); input a,b,c; output d0,d1,d2,d3,d4,d5,d6,d7; wire A,B,C; not not1(A,a); not not2(B,b); not not3(C,c); wire D0,D1,D2,D3,D4,D5,D6,D7; and dd0(D0,A,B); and ddd0(d0,D0,C); and dd1(D1,A,B); and ddd1(d1,D1,c); and dd2(D2,A,b); and ddd2(d2,D2,C); and dd3(D3,A,b); and ddd3(d3,D3,c); and dd4(D4,a,B); and ddd4(d4,D4,C); and dd5(D5,a,B); and ddd5(d5,D5,c); and dd6(D6,a,b); and ddd6(d6,D6,C); and dd7(D7,a,b); and ddd7(d7,D7,c); endmodule module testbench_decode(); reg x,y,z; wire a,b,c,d,e,f,g,h; decoder dec(x,y,z,a,b,c,d,e,f,g,h); initial begin x=0; y=0; z=0; #50 x=0; y=0; z=1; #50 x=0; y=1; z=0; #50 x=0; y=1; z=1; #50 x=1; y=0; z=0; #50 x=1; y=0; z=1; #50 x=1; y=1; z=0; #50 x=1; y=1; z=1; #50; end endmodule;

Output:

Logi Sim Output:

Task 3(Gate level):

Description:

In this task, I used gate level system. By initializing 3 inputs and 9 outputs which I taken for 8 decoders and one final output for multiplexer. So, created first decoder then applied or gate implementation to get desired output.

Code:

module decoder(a,b,c,d0,d1,d2,d3,d4,d5,d6,d7,out); input a,b,c; output d0,d1,d2,d3,d4,d5,d6,d7,out; wire A,B,C; not not1(A,a); not not2(B,b); not not3(C,c); wire D0,D1,D2,D3,D4,D5,D6,D7; and dd0(D0,A,B); and ddd0(d0,D0,C); and dd1(D1,A,B); and ddd1(d1,D1,c); and dd2(D2,A,b); and ddd2(d2,D2,C); and dd3(D3,A,b); and ddd3(d3,D3,c); and dd4(D4,a,B); and ddd4(d4,D4,C); and dd5(D5,a,B); and ddd5(d5,D5,c); and dd6(D6,a,b); and ddd6(d6,D6,C); and dd7(D7,a,b); and ddd7(d7,D7,c); or OUT(out,d0,d1,d2,d3,d4,d5,d6,d7); endmodule module testbench_multiplexer(); reg x,y,z; wire a,b,c,d,e,f,g,h,O; decoder dec(x,y,z,a,b,c,d,e,f,g,h,O); initial begin x=0; y=0; z=0; #50 x=0; y=0; z=1; #50 x=0; y=1; z=0; #50 x=0; y=1; z=1; #50 x=1; y=0; z=0; #50 x=1; y=0; z=1; #50 x=1; y=1; z=0; #50 x=1; y=1; z=1; #50; end endmodule;

Output:

Task 3(Data Flow):

Description:

In this task, I used Data Flow system. By initializing 3 inputs and 9 outputs which I taken for 8 decoders and one final output for multiplexer. So, created first decoder then applied or gate implementation to get desired output.

Code:

module multiplexer(a,b,c,d0,d1,d2,d3,d4,d5,d6,d7,out); input a,b,c; output d0,d1,d2,d3,d4,d5,d6,d7,out; assign d0 = ~a&~b&~c; assign d1 = ~a&~b&c; assign d2 = ~a&b&~c; assign d3 = ~a&b&c; assign d4 = a&~b&~c; assign d5 = a&~b&c; assign d6 = a&b&~c; assign d7 = a&b&c; assign out = d0¦d1¦d2¦d3¦d4¦d5¦d6¦d7; endmodule module testbench_decode(); reg x,y,z; wire a,b,c,d,e,f,g,h,O; multiplexer dec(x,y,z,a,b,c,d,e,f,g,h,O); initial begin x=0; y=0; z=0; #50 x=0; y=0; z=1; #50 x=0; y=1; z=0; #50 x=0; y=1; z=1; #50 x=1; y=0; z=0; #50 x=1; y=0; z=1; #50 x=1; y=1; z=0; #50 x=1; y=1; z=1; #50; end endmodule;

Output:

Logi Sim Output: