Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#06)

Write Lab Title Here

Student Name:         Abdul Ahad Minhas     Roll No: 24i-6531      Section: CE-A

Date performed : 27 Sep, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

This task demonstrates how a procedure can modify registers and why important register values must be preserved. The stack is used to save and restore s0 so that the correct result is obtained after returning from the procedure.

Code:

.text main: li a0, 1 li s0, 2500 jal ra, procedure add t1, s0, a0 li a0, 1 mv a1, t1 ecall li a0, 10 ecall procedure: addi sp, sp, -4 sw s0, 0(sp) li s0, 3 li t0, 1000 add a0, s0, t0 lw s0, 0(sp) addi sp, sp, 4 jalr zero, ra, 0

Output:

Task 2:

Description:

This task demonstrates how registers can be stored temporarily on the stack using sw and retrieved using lw. The stack pointer sp is adjusted to allocate and release the required memory.

Code:

.text main: addi s0, zero, 5 addi t1, zero, 10 addi t0, zero, 15 addi sp, sp, -12 sw s0, 0(sp) sw t0, 4(sp) sw t1, 8(sp) lw t4, 8(sp) lw t2, 4(sp) lw t3, 0(sp) addi sp, sp, 12 li a0, 10 ecall

Output:

Task 3:

Description:

This task implements the given EqSolver(a,b) function as a RISC-V procedure. Arguments are passed through a0 and a1, and the calculated result (a+b)(a-b) is returned through a0.

Code:

.text main: li a0, 7 li a1, 3 jal ra, EqSolver mv t0, a0 li a0, 1 mv a1, t0 ecall li a0, 10 ecall EqSolver: add t0, a0, a1 sub t1, a0, a1 mul a0, t0, t1 jalr zero, ra, 0

Output:

Task 4:

Description:

This task calculates the area and perimeter of a square using two separate procedures, AreaFinder and PerimeterFinder. Both procedures are called from the main program and return their results through a0.

Code:

.text main: li a0, 5 jal ra, AreaFinder mv t0, a0 li a0, 5 jal ra, PerimeterFinder mv t1, a0 li a0, 1 mv a1, t0 ecall li a0, 1 mv a1, t1 ecall li a0, 10 ecall AreaFinder: mul a0, a0, a0 jalr zero, ra, 0 PerimeterFinder: li t0, 4 mul a0, a0, t0 jalr zero, ra, 0

Output:

Task 5:

Description:

This task implements a procedure to calculate the polynomial x³ + 5x² + 6x. Different RISC-V multiplication and addition instructions are used to calculate the individual terms and produce the final result.

Code:

.text main: li a0, 2 jal ra, EqSolver mv t0, a0 li a0, 1 mv a1, t0 ecall li a0, 10 ecall EqSolver: mul t0, a0, a0 mul t1, t0, a0 li t2, 5 mul t3, t0, t2 li t2, 6 mul t4, a0, t2 add t5, t1, t3 add a0, t5, t4 jalr zero, ra, 0

Output:

Task 6:

Description:

This task implements multiple functions and uses conditional statements to decide which procedure should be called. Depending on the values of n and k, the program executes F1, F2, or F3.

Code:

.text main: li a0, 4 # n li a1, 2 # k li t0, 1 beq a1, t0, CALL_F1 bgt a0, a1, CALL_F2 j CALL_F3 CALL_F1: jal ra, F1 j PRINT CALL_F2: jal ra, F2 j PRINT CALL_F3: jal ra, F3 PRINT: mv t1, a0 li a0, 1 mv a1, t1 ecall li a0, 10 ecall F1: mul t0, a0, a0 mul t1, a1, a1 add a0, t0, t1 jalr zero, ra, 0 F2: mul t0, a0, a0 mul a0, t0, a0 jalr zero, ra, 0 F3: addi a0, a0, 5 jalr zero, ra, 0

Output:

Task 7:

Description:

This task uses conditional branching to select one of three functions: fun1, fun2, or fun3. The selected function performs square, cube, or addition operations according to the given conditions.

Code:

.text main: li a0, 4 # n li a1, 4 # k beq a1, zero, CALL_FUN1 beq a0, a1, CALL_FUN2 j CALL_FUN3 CALL_FUN1: jal ra, fun1 j PRINT CALL_FUN2: jal ra, fun2 j PRINT CALL_FUN3: jal ra, fun3 PRINT: mv t0, a0 li a0, 1 mv a1, t0 ecall li a0, 10 ecall fun1: mul a0, a0, a0 jalr zero, ra, 0 fun2: mul t0, a0, a0 mul a0, t0, a0 jalr zero, ra, 0 fun3: addi a0, a0, 1 jalr zero, ra, 0

Output:

Task 8: Without Recursion

Description:

This task calculates the Fibonacci sequence using an iterative loop instead of recursion. The values of consecutive Fibonacci numbers are updated using registers until the required nth value is obtained.

Code:

.text main: li a0, 10 jal ra, fib mv t0, a0 li a0, 1 mv a1, t0 ecall li a0, 10 ecall fib: beq a0, zero, FIB_ZERO li t0, 0 li t1, 1 li t2, 2 FIB_LOOP: bgt t2, a0, FIB_DONE add t3, t0, t1 mv t0, t1 mv t1, t3 addi t2, t2, 1 j FIB_LOOP FIB_ZERO: li a0, 0 jalr zero, ra, 0 FIB_DONE: mv a0, t1 jalr zero, ra, 0

Output:

Task 8: With Recursion

Description:

This task implements the Fibonacci function using recursive procedure calls. The stack is used to preserve the return address and required values during multiple nested function calls.

Code:

.text main: li a0, 6 jal ra, fib mv t0, a0 li a0, 1 mv a1, t0 ecall li a0, 10 ecall fib: beq a0, zero, FIB_ZERO li t0, 1 beq a0, t0, FIB_ONE addi sp, sp, -12 sw ra, 8(sp) sw a0, 4(sp) addi a0, a0, -1 jal ra, fib sw a0, 0(sp) lw a0, 4(sp) addi a0, a0, -2 jal ra, fib lw t0, 0(sp) add a0, t0, a0 lw ra, 8(sp) addi sp, sp, 12 jalr zero, ra, 0 FIB_ZERO: li a0, 0 jalr zero, ra, 0 FIB_ONE: li a0, 1 jalr zero, ra, 0

Output: